江西省永泰化工有限公司

JIANGXI YONGTAI CHEMICAL CO., LTD

Add:west zone. Scientific and Industrial park .luXi county, PingXiang City.

Tel:0086-799-7586358/939   Fax:0086-799-7586721

P1

Terphthalic Acid<MSDS>

| INFOSATE: | PTA06717 | ISUE/DATE | 2006/07 | STATUE | ISSUEDBYJXYONGTAI |
| --- | --- | --- | --- | --- | --- |

Not classified as hazardous according to criteria of U N

| Company Name | JiangXi YongTai Chemical Co.,Ltd |
| --- | --- |
| Address | West Zone. Scientific and Industrial Park.LuXi County. PingXiang City. JiangXi Province,China |
| Emergency Tel | 0086-799-7586939 |
| Tel/Fax | Tel:0086-799-7586358/939 Fax:0086-799-7586721 |
| Other | No data |

| Physical Data |  |
| --- | --- |
| Appearance | White crystals or powder. Odourless or a slight acetic acid odour |
| Vapour Pressure | ＜0.0013Kpa |
| Melting Point | Not available |
| Boiling Point | Not available |
| Flash Point | 260℃<open up> |
| Solubility in Water | Insoluble |
| Specific Gravity | 1.522 |
| Flamm. Limit LEL | Not applicable |
| Flamm. Limit UEL | Not applicable |
| Provided by JiangXi Chem. Co., Ltd. We keep the right to amendment |  |

江西省永泰化工有限公司

JIANGXI YONGTAI CHEMICAL CO., LTD

Add:west zone. Scientific and Industrial park .luXi county, PingXiang City.

Tel:0086-799-7586358/939   Fax:0086-799-7586721

P2

Other Properties

| Autoignition Temp. | 496℃ |
| --- | --- |
| Stability | Stable under normal conditions |
| Haz. Polymerization | Will not occur |
| Materials to Avoid | Incompatible with alkalis and strong oxidising agents |
| Other Information | Relative Vapour Density: 5.74 (air=1) |

| Health Effects |  |
| --- | --- |
| Acute - Swallowed | No adverse health effects expected, however large amounts may cause nausea and vomiting. |
| Acute - Eye | May cause mild eye irritation due to abrasive effects. |
| Acute - Skin | May cause mild irritation |
| Acute - Inhaled | May cause mild respiratory irritation |
| Hazards Identification | No data |
| Chronic | None known |

| First Aid |  |
| --- | --- |
| Swallowed | DNT produce vomiting. Wash out mouth with water. Do not give anything by mouth to an unconscious person. Seek immediate medical attention. |
| Eye | If contact with the eye(s) occur, wash with running water holding eyelid(s) open. Take care not to rinse contaminated water into the non-affected eye. In all cases of eye contamination it is a sensible precaution to seek medical advice. |

江西省永泰化工有限公司

JIANGXI YONGTAI CHEMICAL CO., LTD

Ad:west zone. Scientific and Industrial park .luXi county, PingXiang City.

Tel:0086-799-7586358/939   Fax:0086-799-7586721

P3

| Skin | Wash affected area thoroughly with soap and water. If symptoms develop seek medical attention |
| --- | --- |
| Inhaled | Remove the source of contamination or move the victim to fresh air. Ensure airways are clear and have qualified person give oxygen through a face mask if breathing is difficult. If symptoms develop seek medical attention |
| First Aid Facilities | Eye wash bottle and normal washroom facilties |

| Personal Protection |  |
| --- | --- |
| Eye Protection | Safety glasses with side shields or goggles should be worn |
| Eng. Controls | Ensure ventilation is adequate to maintain air concentrations of below exposure standads. Prevent formation of dusts. Use with local exhaust ventilation. |

| Protectors for Industrial Applications |  |
| --- | --- |
| Glove Type | Wear gloves of impervious material:Occupational protective gloves - Selection, use and maintenance. Final choice of appropriate glove type will vary according to individual circumstances. This can include methods of handling, and engineering controls as determined by appropriate risk assessments. |
| Clothing | Suitable workwear should be worn to protect personal clothing, eg cotton overalls buttoned at neck and wrist. When large quantities are handled the use of plastic aprons and rubber boots is recommended. |
| Footwear | Safety boots. |
| Work/Hygienic Practices | Ensure a high level of personal hygiene is maintained when using this product. Always wash hands before eating, drinking, smoking or using the toilet. |

江西省永泰化工有限公司

JIANGXI YONGTAI CHEMICAL CO., LTD

Ad:west zone. Scientific and Industrial park .luXi county, PingXiang City.

Tel:0086-799-7586358/939   Fax:0086-799-7586721

P4

| Storage and Transport |  |
| --- | --- |
| Storage Precautions | Store in a dry cool place out of direct sunlight. Store away from alkalis and strong oxidising agents. Keep containers closed at all times. Static electricity build up is possible when handling, therefore effective earthing of equipment is essential. |
| Transport | Not classified as a Dangerous Good according to the U N for the Transport of Dangerous Goods by Road and Rail. |
| Other Storage Info. | Dusty conditions must be avoided. This can be achieved by good housekeeping. |
| Proper Shipping Name | None Allocated |

| Spills and Disposal |  |  |
| --- | --- | --- |
| Spills & Disposal | Remove all sources of heat. Increase ventilation. Wear sufficient respiratory protection and full protective clothing to minimise skin and eye exposure. Vacuum or sweep up material avoiding dust generation or dampen spilled material with water to avoid airborne dust. Seal all wastes in vapour tight labelled plastic containers for eventual disposal. If large quantities of this material enter the waterways contact the Environmental Protection Authority, or your local Waste Management Authority. |  |
| Disposal | Dispose of according to relevant government regulations. |  |
| Provided by JiangXi Chem. Co., Ltd. We keep the right to amendment |  |  |

江西省永泰化工有限公司

JIANGXI YONGTAI CHEMICAL CO., LTD

Ad:west zone. Scientific and Industrial park .luXi county, PingXiang City.

Tel:0086-799-7586358/939   Fax:0086-799-7586721

P5

| Fire/Explosion Hazard |  |
| --- | --- |
| Fire/Explos. Hazard | Combustible solid. This product in sufficient quantity and reduced particle size is capable of creating a dust explosion |
| Hazardous Combustion Products | On burning will emit toxic fumes including those of carbon dioxide and carbon monoxide |
| Fire Fighting Precautions | Fire fighters should wear full protective clothing and self-contained breathing apparatus (SCBA) operated in positive pressure mode. |
| Extinguishing Media | Water fog (or if unavailable fine water spray), foam, dry agent (carbon dioxide, dry chemical powder). Water foam may cause frothing which may be violent. |
| Hazardous Reaction | None known |
| Hazchem Code | None Allocated |

Other information

The data in this Material Safety Data Sheet is believed to be correct. However .since conditions of ues are outside our control is should not taken as a warranty of representation for which JiangXi YongTai Chemical Co.,LTD . assumes legal responsibility . This information is provided solely for your consideration, investigation and verification .

| Compositon/Information on Ingredients |  |  |  |
| --- | --- | --- | --- |
| Ingredients | Chemical Formula | CAS NO. | HS Code |
| Terphthalic acid | C8H6O4 | 100-21-0 | 29173610 |