JIANGXI YONG TAI CHEMICAL  CO., LTD.

- Luxi county,Pingxiang City,Jiangxi province, China.

Tel: +86-799-7586939   Fax: +86-799 –7586721

www.jxyongtai.com

### MATERIAL SAFETY DATA SHEET

### Sodium Percarbonate

Section 1. Chemical Product and Supplier Identification

Product Name : Sodium Percarbonate

Synonyms : Sodium Carbonate Peroxyhydrate, PCS, Sodium Carbonate Peroxide

FORMULA ITEM : 2NA2CO3.3H2O2

2. Composition/Information on Ingredients

This product has not been tested as a whole for health effects on animals or humans. According to the OSHA Hazardous Communications Standard (29 CFR 1910.1200) any hazardous ingredients that comprise >1.0% of the material and any

carcinogenic ingredients that comprise >0.1% must be listed on the MSDS. When individually tested at 100% ingredients in the make-up of this product posed the following health hazards. This product is further diluted 1 tablespoon with 5 gallons or 19 litres of water when used.

SECTION II a: HAZARDOUS INGREDIENTS >1.0%

| Chemical Name | CAS # | PEL | TLV |
| --- | --- | --- | --- |
| Sodium Percarbonate | 15630-89-4 | NA | NA |

SECTION II b: CARCINOGENIC INGREDIENTS AT >0.1%

None.

3. Hazards Identification

Emergency Overview

& Oxidizing agent, contact with other material may cause fire

& May be harmful or fatal if swallowed

& May cause severe eye and respiratory tract irritation or burns

& May cause skin irritation

& Does not present any significant hazard for the environment

## Potential Health Effects

General:  Irritating to mucous membrane, eyes, and skin.

Inhalation: Irritating to the respiratory tract. Coughing, sneezing, difficulty breathing and sore throat.

Eye contact: May cause irritation to the eyes, including pain, redness and

reversible damage.

Skin contact:  Slight irritation.

Ingestion:  Vomiting and diarrhea.

4.First-aid Measures

EYES: Flush with water for at least 15 minutes. If irritation persists consult physician.

INGESTION: Drink 1 to 2 glasses of water. Consult physician.

INHALED: Remove to fresh air. If symptoms persist or are severe seek prompt medical assistance.

SKIN: Remove contaminated clothing. Thoroughly wash exposed area with soap and water.

PHYSICIAN: Treatment based on judgment of attending physician.

5. Fire Fighting Measure

Flash Point:  Not applicable

Flammability: Not applicable

Ignition Temperature: Not applicable

Danger of Explosion: Non-explosive

Extinguishing Media: Water

## Fire Hazards

Oxidizer. Storage vessels involved in a fire may vent gas or rupture due to internal pressure. Damp material may decompose exothermically and ignite combustibles. Oxygen release due to exothermic decomposition may support combustion. May ignite other combustible materials. Avoid contact with incompatible materials such as heavy metals, reducing agents, acids, bases, combustibles (wood, papers, cloths etc.). Thermal decomposition releases oxygen and heat. Pressure bursts may occur due to gas evolution. Pressurization if confined when heated or decomposing. Containers may burst violently.

## Fire-Fighting Measures

& Evacuate all non-essential personnel

& Wear protective clothing and self-contained breathing apparatus

& Remain upwind of fire to avoid hazardous vapors and decomposition products

& Use water spray to cool fire-exposed containers

6. Accidental Release Measures

Spill Clean-up Procedures

& Oxidizer. Eliminate all sources of ignition. Evacuate unprotected personnel from equipment recommendations found in Section 8. Never exceed any occupational exposure limit.

& Shovel or sweep material into plastic bags or vented containers for disposal. Do not return spilled or contaminated material to inventory.

& Flush remaining area with water to remove trace residue and dispose of properly.

& Avoid direct discharge to sewers and surface waters. Notify authorities if entry occurs.

& Do not touch or walk through spilled material. Keep away from combustibles (wood, paper, oils, etc.). Do not return any product to container because of the risk of contamination.

7.Handling and Storage

Storage

& Oxidizer. Store in a cool, well ventilated area away from all source of ignition and out of direct sunlight. Store in a dry location away from heat. Store at temperatures less than 40°C.

& Keep away from incompatible materials. Keep containers tightly closed. Do not store in unlabeled or mislabeled containers.

& Protect from moisture. Do not store near combustible materials. Keep containers well sealed, seal only with original vent cap. Ensure pressure relief and adequate ventilation.

& Store separately from organics and reducing materials. Avoid contamination which may lead to decomposition.

## Handling

& Avoid contact with eyes, skin, and clothing. Use with adequate ventilation.

& Do not swallow. Avoid breathing vapors, mists, or dust. Do not eat, drink, or smoke in work area.

&Prevent contact with combustible or organic materials.

& Label containers and keep them tightly closed when not in use.

& Wash thoroughly after handling.

8.Exposure Controls/Personal Protection

## Engineering Controls

General room ventilation is required. Local exhaust ventilation, process enclosures or other engineers controls may be needed to maintain airborne levels below recommended exposure limits. Avoid creating dust or mist. Maintain adequate ventilation. Do not use in closed or confined spaces. Keep levels below exposure limits. To determine exposure levels, monitoring should be performed regularly.

## Respiratory Protection

For many conditions, no respiratory protection may be needed; however, in dusty or

unknown atmospheres or when exposures exceed limit values, wear a NIOSH approved respirator.

## Eye/Face Protection

Wear chemical safety goggles and a full face shield while handling this product.

## Skin Protection

Prevent contact with this product. Wear gloves and protective clothing depending on

condition of use. Protective gloves: Chemical-resistant (Recommended materials: PVC, neoprene or rubber)

## Other Protective Equipment

Eye-wash station

Safety shower

Impervious clothing

Rubber boots

## General Hygiene Considerations

Wash with soap and water before meal times and at the end of each work shift. Good

manufacturing practices require gross amounts of any chemical be removed from skin as soon as practical, especially before eating or smoking.

9. Physical and Chemical Properties

APPEARANCE AND ODOR:  White free flowing powder odorless.

WATER SOLUBILITY, % BY WT:  Approx. 14

LOOSE BULK DENSITY:  1.05 g/cc

%VOLATILE BY WT:  negligible

pH of a 1% solution:  10.0 - 11.0

10. Stability and Reactivity

Stability

Stable under normal conditions

Conditions to Avoid

Water

Acids

Bases

Salts of heavy metals

Reducing agents

Organic materials

Flammable substances

Hazardous Decomposition Products

Oxygen. Contamination with many substances will cause decomposition. The rate of

decomposition increases with increasing temperature and may be very vigorous with rapid generation of large volume of oxygen and steam.

11.Toxicological Information

Sodium Percarbonate: Oral rat LD50 is 1034 g/kg, Dermal rabbit LD50 is >2000 mg/kg.

12. Ecological Information

Ready to use solution is biodegradable. No other information is available.

13. Disposal Considerations

## Waste Treatment

Dispose of in an approved waste facility operated by an authorized contractor in compliance with local regulations.

## Package Treatment

The empty and clean containers are to be recycled or disposed of in conformity with local regulations.

14. Transport Information

Proper Shipping Name: Oxidizing solid, n.o.s., (sodium carbonate peroxyhydrate)

UN Number: UN3378

Hazard Class: 5.1

Labels: 5.1 (Oxidizer)

Packing Group: II

15. Regulatory Information

SARA Section……………….……... Yes

SARA (313) Chemicals……………. No

EPA TSCA Inventory………………. Appears

Canadian WHMIS Classification….. C, D2B

Canadian DSL……………………... Appears

EINECS Inventory…………………. Appears

16. Other Information

One should always use chemicals with awareness of potential hazards, known and unknown, and their effect on the human system. We urge users to minimize employee exposure to all chemicals, including this product and to keep the level of exposure commensurate with good industrial hygiene practices.

## DISCLAIMER OF LIABILITY

The manufacturer and seller warrants that this product conforms to its standard specifications when used according to directions. As the conditions or methods of use are beyond our control, we do not assume any responsibility and expressly disclaim any liability for use of this product. Information contained herein is believed to be true and accurate but all statements or suggestions are made without warranty, expressed or implied, regarding accuracy of the information, the hazards connected with the use of the material and the results to be obtained from the use thereof.